FLIPFOCUS

A

# MINOR PROJECT-II REPORT

Submitted in partial fulfillment of the requirements for the award of degree

BACHELOR OF TECHNOLOGY

In

Computer Science and Engineering –

Artificial Intelligence and Machine Learning

Submitted by

| NIPUN LAKHERA | 0002AL231047 |
| --- | --- |
| SAHIL RAIKWAR | 0002AL231055 |
| MO ZAID SHEIKH | 0002AL231042 |
| SHIVANSH NIGAM | 0002AL231062 |

Under the guidance of

Mr. Mahendra K Ahirwar

(GUIDE)

Mr. Sanjay Singh

(CO-GUIDE)

SCHOOL OF INFORMATION TECHNOLOGY

RAJIV GANDHI PROUDYOGIKI VISHWAVIDYALAYA

BHOPAL (M.P.)

JANUARY-JUNE 2026

SCHOOL OF INFORMATION TECHNOLOGY

RAJIV GANDHI PROUDYOGIKI VISHWAVIDYALAYA

BHOPAL (M.P.)

# CERTIFICATE

This is to certify that Nipun Lakhera, Sahil Raikwar, Mo Zaid Sheikh, Shivansh Nigam of B.Tech third year, Computer Science and Engineering – Artificial Intelligence and Machine Learning have completed their Minor Project-II entitled “FLIPFOCUS: A Physical IoT-Based Productivity and Deep Work Logging Ecosystem” during the period from January 2026 to June 2026 under the guidance and supervision.

We approve the project for the submission for partial fulfillment of the requirements for the award of the degree of Bachelor of Technology in Computer Science and Engineering – Artificial Intelligence and Machine Learning.

Mr. Mahendra K Ahirwar

Project Guide

Dr. Jitendra Agrawal

Director

SOIT, RGPV

Mr. Sanjay Singh

Project Co-Guide

SCHOOL OF INFORMATION TECHNOLOGY

RAJIV GANDHI PROUDYOGIKI VISHWAVIDYALAYA

BHOPAL (M.P.)

# APPROVAL CERTIFICATE

This is hereby certify that Minor Project-II entitled “FLIPFOCUS: A Physical IoT-Based Productivity and Deep Work Logging Ecosystem” being submitted by Nipun Lakhera (0002AL231047), Sahil Raikwar (0002AL231055), Mo Zaid Sheikh (0002AL231042), and Shivansh Nigam (0002AL231062) of B.Tech third year, Computer Science and Engineering – Artificial Intelligence and Machine Learning in full to the School of Information Technology, RGPV, Bhopal.

External Examiner

Date:

Internal Examiner

Date:

SCHOOL OF INFORMATION TECHNOLOGY

RAJIV GANDHI PROUDYOGIKI VISHWAVIDYALAYA

BHOPAL (M.P.)

# DECLARATION BY CANDIDATE

We, hereby declare that the work which is presented in the Minor Project-II entitled “FLIPFOCUS: A Physical IoT-Based Productivity and Deep Work Logging Ecosystem” submitted in partial fulfillment of the requirements for the award of the degree of Bachelor of Technology in Computer Science and Engineering – Artificial Intelligence and Machine Learning has been carried out at School of Information Technology, RGPV, Bhopal and is an authentic record of our work carried out under the guidance of Mr. Mahendra K Ahirwar (Project guide) and Mr. Sanjay Singh (Project co-guide), School of Information Technology, RGPV, Bhopal.

The matter in this project has not been submitted by us for the award of any other degree.

| NIPUN LAKHERA | 0002AL231047 |
| --- | --- |
| SAHIL RAIKWAR | 0002AL231055 |
| MO ZAID SHEIKH | 0002AL231042 |
| SHIVANSH NIGAM | 0002AL231062 |

SCHOOL OF INFORMATION TECHNOLOGY

RAJIV GANDHI PROUDYOGIKI VISHWAVIDYALAYA

BHOPAL (M.P.)

# ACKNOWLEDGEMENT

After the completion of Minor Project-II, words are not enough to express our feelings about all those who helped us to reach our goal, feeling above all this is our indebtedness to the almighty for providing us this moment in life. First and foremost, we take this opportunity to express out deep regards and heartfelt gratitude to our project guide Mr. Mahendra K Ahirwar, and project co-guide Mr. Sanjay Singh of School of Information Technology, RGPV, Bhopal for their inspiring guidance and timely suggestions in carrying out our project successfully. They have also been a constant source of inspiration for us.

We are extremely thankful to Dr. Jitendra Agrawal, Director, School of Information Technology, RGPV, Bhopal for his cooperation and motivation during the project. We would also like to thank all the teachers of our department for providing invaluable support and motivation. We are also grateful to our family for their help and cooperation throughout this work.

| NIPUN LAKHERA | 0002AL231047 |
| --- | --- |
| SAHIL RAIKWAR | 0002AL231055 |
| MO ZAID SHEIKH | 0002AL231042 |
| SHIVANSH NIGAM | 0002AL231062 |

# TABLE OF CONTENTS

ABSTRACT	7

LIST OF FIGURES	8

LIST OF TABLES	9

LIST OF ABBREVIATIONS	10

CHAPTER 1: INTRODUCTION	11

1.1 INTRODUCTION	11

1.2 PURPOSE	11

1.3 OBJECTIVES	12

1.4 INTERFACE	12

1.5 DESIGN AND IMPLEMENTATION CONSTRAINTS	12

CHAPTER 2: SOFTWARE & HARDWARE REQUIREMENTS	13

2.1 SOFTWARE REQUIREMENTS	13

2.2 HARDWARE REQUIREMENTS	13

CHAPTER 3: PROBLEM DESCRIPTION	16

CHAPTER 4: LITERATURE SURVEY	17

CHAPTER 5: SOFTWARE REQUIREMENT SPECIFICATION (SRS)	18

5.1 INTRODUCTION	18

5.2 FUNCTIONAL REQUIREMENTS	18

5.2.1 USER CLASS-1 (PRIMARY LEARNER)	18

5.2.2 USER CLASS-2 (SUPERVISOR / EVALUATOR)	20

5.3 NON-FUNCTIONAL REQUIREMENTS	20

CHAPTER 6: DESIGN AND IMPLEMENTATION	22

6.1 SYSTEM ARCHITECTURE DESIGN	22

6.2 TABLE STRUCTURE (FIREBASE JSON SCHEMA)	23

6.3 USE CASE DIAGRAM	23

6.4 DATA FLOW DIAGRAMS (DFD)	24

6.5 ESP32 FIRMWARE SOURCE CODE (main.ino)	25

CHAPTER 7: RESULT & ANALYSIS	45

7.1 ANALOG STABILITY & HYSTERESIS ANALYSIS	45

7.2 SYNC LATENCY COMPARISONS	45

7.3 PROJECT BUDGET COST ANALYSIS	45

CHAPTER 8: DEPLOYMENT	47

CHAPTER 9: CONCLUSION & FUTURE WORK	48

9.1 CONCLUSION	48

9.2 FUTURE WORK	48

APPENDIX-1	GLOSSARY OF TERMS	49

REFERENCES	50

SCHOOL OF INFORMATION TECHNOLOGY

RAJIV GANDHI PROUDYOGIKI VISHWAVIDYALAYA

BHOPAL (M.P.)

# ABSTRACT

In the contemporary academic and professional landscape, maintaining cognitive focus and managing attention has become a primary challenge. Traditional software-based productivity trackers often introduce interface friction and digital distractions, which disrupt flow states. To address these issues, this project introduces "FlipFocus," a physical Internet of Things (IoT) productivity tracker and real-time visualization ecosystem designed to automate focus logging.

The system uses an ESP32 microcontroller that interfaces with a rotary potentiometer (and an MPU6050 accelerometer) to map physical states to four distinct productivity profiles: Assignment, Exam Prep, Core Coding, and Break. The hardware runs a non-blocking execution loop that applies a 64-sample low-pass averaging filter to eliminate analog noise, and implements a dynamic hysteresis algorithm to prevent zone flickering near transitions. To ensure reliability in university environments with unstable WiFi, the system features a dual-mode communication pipeline: a cloud sync mode using the Firebase Realtime Database and an offline local mode using the browser Web Serial API over USB.

Furthermore, to protect the ESP32's internal flash memory from wear due to frequent writes during zone transitions, a Non-Volatile Storage (NVS) wear-mitigation algorithm is implemented to restrict write cycles. The entire prototype is built for ₹940, demonstration-ready, and optimized to help users build consistent focus habits. The user can easily interact with our web application and get benefited without wasting time in offline process.

# LIST OF FIGURES

| Figure No. | Title of Figure | Page No. |
| --- | --- | --- |
| 1.1 | FlipFocus System Architecture Overview | 11 |
| 2.1 | ESP32 development board pinout configuration | 13 |
| 2.2 | I2C 16x2 LCD layout and custom character blocks | 14 |
| 6.1 | Hardware and Software Interaction Block Diagram | 22 |
| 6.3 | System Use Case Diagram | 23 |
| 6.4 | Level-0 Data Flow Diagram | 24 |
| 6.5 | Level-1 Data Flow Diagram | 25 |

# LIST OF TABLES

| Table No. | Title of Table | Page No. |
| --- | --- | --- |
| 2.1 | Software Development Environment Specifications | 13 |
| 2.2 | Hardware Component Specifications | 15 |
| 5.1 | State Transition Mapping Parameters | 18 |
| 7.1 | ADC Input Mapping and Hysteresis Thresholds | 45 |
| 7.2 | Operational Cost Breakdown | 46 |

# LIST OF ABBREVIATIONS

| Acronym | Full Form |
| --- | --- |
| ADC | Analog-to-Digital Converter |
| API | Application Programming Interface |
| BER | Bit Error Rate |
| BOM | Bill of Materials |
| BPSK | Binary Phase Shift Keying |
| BSS | Basic Service Set |
| BSSID | Basic Service Set Identifier |
| CCITT | Comité Consultatif International Téléphonique et Téléphonique |
| CCK | Complementary Code Keying |
| CF | Contention Free |
| CSS | Cascading Style Sheets |
| DFD | Data Flow Diagram |
| ESP32 | Espressif System 32 Microcontroller |
| HTML | HyperText Markup Language |
| I2C | Inter-Integrated Circuit |
| IMU | Inertial Measurement Unit |
| IoT | Internet of Things |
| IST | India Standard Time |
| JS | JavaScript |
| LCD | Liquid Crystal Display |
| NTP | Network Time Protocol |
| NVS | Non-Volatile Storage |
| RTDB | Real-Time Database |
| SDLC | Software Development Life Cycle |
| SQL | Structured Query Language |
| SRS | Software Requirements Specification |
| UML | Unified Modeling Language |
| USB | Universal Serial Bus |

# CHAPTER 1: INTRODUCTION

## 1.1 INTRODUCTION

In an era dominated by digital notifications and fragmented attention, the ability to engage in concentrated deep work has become a critical competitive advantage. However, most productivity tracking tools are software-based applications that run on the same devices where distractions occur (such as smartphones and laptops). Opening a time-tracking application often introduces cognitive load and exposes the user to notifications, interrupting their focus.

[FIGURE 1.1: FlipFocus System Architecture Overview]

FlipFocus is a physical, Internet of Things (IoT)-based time-tracking and focus logging ecosystem. Shifting the logging interface to a physical object on the desk allows users to log study states with a simple tactile gesture. The system maps the physical states of a rotary potentiometer to four key focus zones: Assignment, Exam Prep, Core Coding, and Break.

## 1.2 PURPOSE

The purpose of the FlipFocus ecosystem is to automate focus logging and provide a frictionless, screen-free way to track time. By mapping physical adjustments to study profiles, the system acts as a physical anchor for attention, helping users enter and maintain deep focus.

The data is synchronized in real-time to a cloud-based web dashboard (via Firebase) and an offline local dashboard (via Web Serial API over USB), providing users with immediate visual feedback on their productivity distribution.

## 1.3 OBJECTIVES

- Frictionless Focus Control: Build a physical device that maps focus states using an analog potentiometer and orientation sensors.

- Signal Filtering: Implement non-blocking digital filtering and hysteresis to ensure stable zone selection.

- Dual Telemetry Pipeline: Provide real-time Firebase cloud synchronization alongside an offline USB connection using the Web Serial API.

- Flash Protection: Build NVS wear-out protection that batches flash memory writes.

- Low-Cost Prototyping: Build the entire physical device within a strict ₹1,000 budget.

## 1.4 INTERFACE

The FlipFocus system features a physical-digital user interface:

- Physical Hardware Interface: Uses an analog potentiometer and an LCD screen to display active states, elapsed session time, and network status icons.

- Digital Web Dashboards: High-performance web applications designed with a dark "Blueprint" aesthetic. The dashboards feature dynamic Chart.js donut visualizations and an interactive Pomodoro countdown timer.

- Audio-Visual Feedback: Plays specific buzzer melodies and displays LCD notifications (like "PING RECEIVED" or "TIME'S UP") during events.

## 1.5 DESIGN AND IMPLEMENTATION CONSTRAINTS

- Hardware Constraints: Sourcing and integration must fit within a ₹1,000 budget. Power must be supplied safely via a standard 5V USB connection.

- Memory Constraints: Minimize the number of write operations to the ESP32's flash memory to prevent hardware wear.

- Connectivity Constraints: The system must function reliably in offline mode when WiFi is unavailable, using a local USB connection.

# CHAPTER 2: SOFTWARE & HARDWARE REQUIREMENTS

## 2.1 SOFTWARE REQUIREMENTS

The software development environment and dependencies for FlipFocus are detailed in Table 2.1:

TABLE 2.1: SOFTWARE DEVELOPMENT SYSTEM SPECIFICATIONS

| Software Tool / Dependency | Version | Purpose |
| --- | --- | --- |
| Arduino IDE | 2.3.2 | Firmware compilation and flashing |
| Firebase ESP Client Library | 4.4.14 | Firebase database connectivity |
| Preferences Library | 2.0.0 | ESP32 Non-Volatile Storage (NVS) |
| Chart.js (CDN) | 4.4.2 | Front-end data visualization |
| Firebase Web SDK (CDN) | 8.10.0 | Real-time client-side database sync |
| Web Serial API | Native | Offline browser-to-hardware communication |

## 2.2 HARDWARE REQUIREMENTS

[FIGURE 2.1: ESP32 development board pinout configuration]

The physical prototype is assembled using the following hardware components:

- ESP32 NodeMCU: A 32-bit dual-core MCU with built-in 2.4GHz WiFi, used to handle sensor inputs, LCD rendering, and network communications.

- MPU6050 IMU: A 3-axis accelerometer and gyroscope module connected via I2C to detect device orientation.

- Rotary Potentiometer: A 10k Ohm linear potentiometer connected to ADC pin 34 for zone selection.

- I2C 16x2 LCD: Uses a PCF8574 expander to display real-time status and timers using only two I2C pins (SDA/SCL).

[FIGURE 2.2: I2C 16x2 LCD layout and custom character blocks]

- Active Buzzer: Pin-driven 5V buzzer used to play alert melodies.

- Status LED: Driven by GPIO 14 to provide visual status confirmation.

TABLE 2.2: Hardware Component Specifications

| Component Name | Operating Voltage | Pins Used | Primary Technical Specification |
| --- | --- | --- | --- |
| ESP32 NodeMCU | 3.3V / 5V | Multiple | Dual-core 240MHz CPU 4MB Flash Wi-Fi & BLE |
| MPU6050 IMU | 3.3V | GPIO 21 (SDA) GPIO 22 (SCL) | 3-axis accelerometer and 3-axis gyroscope |
| Rotary Potentiometer | 0V - 3.3V | GPIO 34 (ADC1_CH6) | 10k Ohm linear analog voltage divider |
| I2C 16x2 LCD | 5V | GPIO 21 (SDA) GPIO 22 (SCL) | PCF8574 expander addressing 0x27 address |
| Active Buzzer | 5V | GPIO 13 | Digital pin-driven high-frequency audio buzzer |
| Status LED | 2.2V - 3.3V | GPIO 14 | 5mm red indicator with 220 Ohm resistor |

# CHAPTER 3: PROBLEM DESCRIPTION

Maintaining sustained focus is a primary challenge in modern academic and professional settings. Users struggle with three core issues:

1. Cognitive Overhead of Manual Logging:

Traditional time tracking (using spreadsheets or manual apps) requires active entry. Users must remember to start and stop timers, introducing cognitive friction that interrupts flow states.

2. Screen-Based Distractions:

Digital productivity tools often run on the same devices where distractions occur. Opening an app to track study time exposes users to notifications and browser tabs, disrupting focus.

3. Campus Network Instability:

In academic environments, WiFi networks are often congested or unstable. Standard cloud-dependent IoT trackers fail to function during network drops, leading to data loss.

4. Microcontroller Flash Memory Wear:

Constantly writing telemetry data directly to a microcontroller's internal flash memory leads to rapid hardware degradation and premature failure.

FlipFocus resolves these issues by providing a physical, screen-free interface under a ₹1,000 budget, backed by offline USB serialization and flash memory wear protection.

# CHAPTER 4: LITERATURE SURVEY

To design the FlipFocus system, research was conducted across four key areas:

1. Tangible User Interfaces (TUI):

Research shows that physical objects (like cubes or dials) on a desk can act as environmental cues that reinforce habits. Physical interactions reduce interface friction compared to digital screens.

2. Embedded Low-Pass Noise Filters:

Microcontroller ADCs are prone to thermal and electrical noise. Standard filtering algorithms, such as multi-sample averaging, are required to stabilize analog signals.

3. Hysteresis Boundary Mapping:

Hysteresis is commonly used in thermostat control to prevent rapid switching when inputs hover near a boundary. Implementing it in firmware stabilizes zone selections.

4. Flash Memory Wear Mitigation:

Microcontroller flash sectors support limited write cycles (approx 100,000). Research indicates that write-batching and duration-checks are required to prevent premature memory failure.

# CHAPTER 5: SOFTWARE REQUIREMENT SPECIFICATION (SRS)

## 5.1 INTRODUCTION

This chapter defines the functional and non-functional software requirements for the FlipFocus system. It outlines user classes, functional interfaces, and performance constraints.

TABLE 5.1: State Transition Mapping Parameters

| Active Zone Index | Productivity Profile | Min ADC Threshold | Max ADC Threshold | NVS Preferences Key | Firebase Node |
| --- | --- | --- | --- | --- | --- |
| 1 | Assignment | 0 | 700 | p1 | assignment |
| 2 | Exam Prep | 700 | 2380 | p2 | examprep |
| 3 | Core Coding | 2380 | 4050 | p3 | corecoding |
| 4 | Total Break | 4050 | 4095 | p4 | totalbreak |

## 5.2 FUNCTIONAL REQUIREMENTS

### 5.2.1 USER CLASS-1 (PRIMARY LEARNER)

R.1: Read and Filter Analog Potentiometer Position

Description:

The system must read the analog position of the potentiometer, filter out ADC noise, apply hysteresis around boundary regions, and map the filtered values to four active study profiles: Assignment, Exam Prep, Core Coding, and Break.

R1.1: Read Raw Analog Input

Input   : Raw analog signal on GPIO 34.

Output  : 12-bit integer conversion (0-4095).

R1.2: Perform Noise Filtering

Input   : 64 raw samples.

Output  : Average digital reading.

R1.3: Apply Hysteresis Mapping

Input   : Average reading, current zone index, and hysteresis boundary offsets (+75/-75 for Zones 1-3, +50/-50 for Zone 4).

Output  : Debounced active zone index (1 to 4) and immediate Firebase/Serial push.

Processing:

The average ADC reading is mapped against the boundary zones. To prevent boundary noise from causing flickering, the boundary shifts by +75 (rising) or -75 (falling) steps depending on the active zone. If a zone change is detected, the timer for the previous zone is updated and NVS writes are executed if the duration was >= 1 second.

R.2: Manage Pomodoro Focus Timer

Description:

The user controls study intervals on the dashboard. Upon completion, the dashboard must play local beeps and trigger the ESP32 hardware alarm.

R2.1: Toggle Timer Countdown

Input   : User click on "▶ Start" / "⏸ Pause" button.

Output  : Active countdown state toggled.

R2.2: Reset Timer

Input   : User click on "↺ Reset" button.

Output  : Timer reset to active preset value.

R2.3: Trigger Hardware Alarm

Input   : Countdown timer reaches 0.

Output  : Dashboard sends "alarm" over Web Serial (Local Mode) or sets `/profiles/triggerPing` to `2` (Cloud Mode).

Processing:

A JavaScript interval decrements the timer. On expiry, the dashboard plays a browser beep, sends the alarm command to the ESP32, and the ESP32 displays "  TIME'S UP!    " while playing a rapid buzzer alert.

### 5.2.2 USER CLASS-2 (SUPERVISOR / EVALUATOR)

R.3: Send Attention Ping Alert

Description:

The supervisor triggers a physical alert on the user's hardware device from the web dashboard.

R3.1: Trigger Ping Command

Input   : Click "🔔 Ping Device" button.

Output  : Dashboard sends `ping` over Web Serial (Local Mode) or sets `/profiles/triggerPing` to `1` (Cloud Mode).

R3.2: Play Alert Melody on Hardware

Input   : ESP32 reads Serial command `ping` or Firebase trigger value of `1`.

Output  : LCD displays "PING RECEIVED!" and plays the "tu-tu-tuuu" alert melody.

Processing:

Upon reading trigger value `1`, the LCD displays the custom notification for 2 seconds and the buzzer plays the "tu-tu-tuuu" melody. The trigger value is then reset to `0`.

## 5.3 NON-FUNCTIONAL REQUIREMENTS

- Latency: Synchronization latency must remain under 2.0 seconds.

- Usability: The web interface must be responsive, adapting to both mobile and desktop screens.

- Reliability: The hardware must function in offline mode during network drops and automatically reconnect when WiFi is restored.

# CHAPTER 6: DESIGN AND IMPLEMENTATION

## 6.1 SYSTEM ARCHITECTURE DESIGN

The FlipFocus system uses a three-tier architecture:

- Physical Hardware: An ESP32 reads potentiometer and accelerometer inputs, runs digital filters, and drives the LCD and buzzer.

- Middleware Database: Firebase Realtime Database handles real-time synchronization using WebSocket streams.

- Web Client: Renders data distributions using Chart.js and runs the Pomodoro manager.

[FIGURE 6.1: Hardware and Software Interaction Block Diagram]

## 6.2 TABLE STRUCTURE (FIREBASE JSON SCHEMA)

Firebase Realtime Database stores data in a flat JSON structure under the `/profiles` node:

- `activeZone` (Integer): Currently active zone index (1 to 4).

- `assignment`, `examprep`, `corecoding`, `totalbreak` (Integers): Cumulative seconds logged per profile.

- `potentiometer` (Integer): Raw filtered analog value from the sensor.

- `triggerPing` (Integer): Alert flag (0 = Idle, 1 = Ping, 2 = Alarm).

## 6.3 USE CASE DIAGRAM

The use-case model defines interactions between actors and the system:

- Primary Learner: Interacts with the physical dial, reads the LCD screen, connects to Web Serial over USB, manages the Pomodoro timer, and views analytics.

- Supervisor: Accesses cloud analytics and sends remote pings to the device.

[FIGURE 6.3: System Use Case Diagram]

## 6.4 DATA FLOW DIAGRAMS (DFD)

- Level-0 DFD: The hardware node acts as the data source, transmitting sensor values and receiving control commands. The web dashboard exchanges JSON data with the database.

[FIGURE 6.4: Level-0 Data Flow Diagram]

- Level-1 DFD: Displays internal processes: (1) acquiring raw sensor data, (2) applying averaging filters, (3) applying hysteresis bounds, (4) incrementing active timers, (5) batching NVS updates, and (6) pushing telemetry updates.

[FIGURE 6.5: Level-1 Data Flow Diagram]

## 6.5 ESP32 FIRMWARE SOURCE CODE (main.ino)

#include <WiFi.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <Firebase_ESP_Client.h>

#include <Preferences.h>

#include <time.h>

// NTP Settings for IST (India Standard Time: UTC+5:30)

const char* ntpServer = "pool.ntp.org";

const long  gmtOffset_sec = 19800; // 5.5 * 3600

const int   daylightOffset_sec = 0;

// Secret credentials - THIS FILE IS NOT COMMITTED TO GITHUB

#include "arduino_secrets.h"

// Provide useful error messages if secrets are missing

#ifndef WIFI_SSID

#error "Please create an arduino_secrets.h file with your WIFI_SSID, WIFI_PASSWORD, API_KEY and DATABASE_URL"

#endif

#include "addons/TokenHelper.h"

#include "addons/RTDBHelper.h"

LiquidCrystal_I2C lcd(0x27, 16, 2);

Preferences preferences;

const int potPin = 34;

const int buzzerPin = 13;

const int ledPin = 14;

FirebaseData fbdo;

FirebaseAuth auth;

FirebaseConfig config;

FirebaseJson json; // Used for batch updates

unsigned long accumulatedTime[5] = {0, 0, 0, 0, 0};

unsigned long zoneStartTime = 0;

int currentZone = 0;

int lastZone = -1;

int lastResetDay = 0;

bool isOfflineMode = false;

// Non-blocking timers

unsigned long lastSyncTime = 0;

const unsigned long syncInterval = 3000;

unsigned long lastNVSSave = 0;           // Timer for flash memory protection

const unsigned long nvsSaveInterval = 300000; // Save to flash once every 5 minutes (300000ms)

unsigned long lastTimeCheck = 0; // For NTP check

unsigned long buzzerStartTime = 0;

bool buzzerActive = false;

int currentPotValue = 0;

int readPotentiometer() {

long sum = 0;

const int samples = 64;

for (int i = 0; i < samples; i++) {

sum += analogRead(potPin);

delayMicroseconds(50);

}

return sum / samples;

}

int getZoneFromADC(int rawValue, int currentActiveZone) {

if (currentActiveZone == 0) {

if (rawValue < 700) return 1;

if (rawValue < 2380) return 2;

if (rawValue < 4050) return 3;

return 4;

}

// Hysteresis logic to prevent jitter

switch (currentActiveZone) {

case 1:

if (rawValue > 775) return 2;

break;

case 2:

if (rawValue < 625) return 1;

if (rawValue > 2455) return 3;

break;

case 3:

if (rawValue < 2305) return 2;

if (rawValue > 4050) return 4;

break;

case 4:

if (rawValue < 4000) return 3;

break;

}

return currentActiveZone;

}

// Custom LCD Emojis/Characters (5x8 Bitmaps)

byte wifiConnected[8] = {

B00000,

B00001,

B00001,

B00011,

B00011,

B00111,

B00111,

B11111

};

byte wifiDisconnected[8] = {

B10001,

B01010,

B00100,

B01010,

B10001,

B00000,

B11111,

B00000

};

byte lockChar[8] = {

B01110,

B10001,

B10001,

B11111,

B11011,

B11011,

B11111,

B00000

};

byte breakChar[8] = {

B01010,

B01010,

B11110,

B10001,

B10001,

B11110,

B00000,

B00000

};

void triggerPingResponse() {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("PING RECEIVED!  ");

lcd.setCursor(0, 1);

lcd.print("by team: overAI ");

// Play tu-tu-tuuu melody

// tu

digitalWrite(buzzerPin, HIGH);

digitalWrite(ledPin, HIGH);

delay(100);

digitalWrite(buzzerPin, LOW);

digitalWrite(ledPin, LOW);

delay(80);

// tu

digitalWrite(buzzerPin, HIGH);

digitalWrite(ledPin, HIGH);

delay(100);

digitalWrite(buzzerPin, LOW);

digitalWrite(ledPin, LOW);

delay(80);

// tuuu

digitalWrite(buzzerPin, HIGH);

digitalWrite(ledPin, HIGH);

delay(350);

digitalWrite(buzzerPin, LOW);

digitalWrite(ledPin, LOW);

delay(2000); // Let user read screen

lcd.clear();

updateLCD();

}

void triggerAlarmResponse() {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("  TIME'S UP!    ");

lcd.setCursor(0, 1);

lcd.print("by team: overAI ");

// Play alarm buzzer tune: 4 short, 1 long

for (int i = 0; i < 4; i++) {

digitalWrite(buzzerPin, HIGH);

digitalWrite(ledPin, HIGH);

delay(80);

digitalWrite(buzzerPin, LOW);

digitalWrite(ledPin, LOW);

delay(50);

}

digitalWrite(buzzerPin, HIGH);

digitalWrite(ledPin, HIGH);

delay(350);

digitalWrite(buzzerPin, LOW);

digitalWrite(ledPin, LOW);

delay(2000); // Let user read screen

lcd.clear();

updateLCD();

}

void setup() {

Serial.begin(115200);

// Load saved times and last reset date from Non-Volatile Storage

preferences.begin("pomodoro", false);

accumulatedTime[1] = preferences.getUInt("p1", 0);

accumulatedTime[2] = preferences.getUInt("p2", 0);

accumulatedTime[3] = preferences.getUInt("p3", 0);

accumulatedTime[4] = preferences.getUInt("p4", 0);

lastResetDay = preferences.getInt("lrd", 0);

preferences.end();

Serial.println("Previous progress loaded from Flash.");

pinMode(potPin, INPUT);

pinMode(buzzerPin, OUTPUT);

pinMode(ledPin, OUTPUT);

lcd.init();

lcd.backlight();

// Register custom characters

lcd.createChar(0, wifiConnected);

lcd.createChar(1, wifiDisconnected);

lcd.createChar(2, lockChar);

lcd.createChar(3, breakChar);

lcd.setCursor(0, 0);

lcd.print("CONNECTING WIFI ");

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

// 4-Second Timeout Loop

unsigned long startAttemptTime = millis();

while (WiFi.status() != WL_CONNECTED && millis() - startAttemptTime < 4000) {

delay(200);

Serial.print(".");

}

lcd.clear();

if (WiFi.status() == WL_CONNECTED) {

lcd.print("WIFI CONNECTED!");

isOfflineMode = false;

// Initialize Time and Firebase ONLY if connected

configTime(gmtOffset_sec, daylightOffset_sec, ntpServer);

config.api_key = API_KEY;

config.database_url = DATABASE_URL;

Firebase.signUp(&config, &auth, "", "");

Firebase.begin(&config, &auth);

Firebase.reconnectWiFi(true);

} else {

lcd.print("OFFLINE MODE ON ");

isOfflineMode = true;

// Just minimal config for later

config.api_key = API_KEY;

config.database_url = DATABASE_URL;

Firebase.begin(&config, &auth);

Firebase.reconnectWiFi(true);

}

delay(1000);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("  FLIPFOCUS v1  ");

lcd.setCursor(0, 1);

lcd.print(isOfflineMode ? " LOCAL TRACKING" : " CLOUD SYNC LIVE");

delay(1500);

lcd.clear();

zoneStartTime = millis();

updateLCD();

}

void loop() {

// Update offline status dynamically

bool wasOffline = isOfflineMode;

isOfflineMode = (WiFi.status() != WL_CONNECTED);

// If we just reconnected online, re-run NTP config to sync time

if (wasOffline && !isOfflineMode) {

Serial.println("Reconnected online. Initializing NTP...");

configTime(gmtOffset_sec, daylightOffset_sec, ntpServer);

}

// Check for day change every 60 seconds if online

if (!isOfflineMode && (millis() - lastTimeCheck > 60000)) {

checkForDayReset();

lastTimeCheck = millis();

}

currentPotValue = readPotentiometer();

int newZone = getZoneFromADC(currentPotValue, currentZone);

// Show the value of potentiometer in serial monitor reading to adjust values for each profile

static unsigned long lastPotPrintTime = 0;

if (millis() - lastPotPrintTime > 500) {

Serial.print("POT_VAL: ");

Serial.print(currentPotValue);

Serial.print(" -> Active Zone: ");

Serial.println(currentZone);

lastPotPrintTime = millis();

}

if (newZone != currentZone) {

unsigned long timeBefore = (currentZone >= 1 && currentZone <= 4) ? accumulatedTime[currentZone] : 0;

updateAccumulatedTime(); // Move progress from current zone to array

bool timeChanged = (currentZone >= 1 && currentZone <= 4) && (accumulatedTime[currentZone] != timeBefore);

currentZone = newZone;

zoneStartTime = millis();

// Trigger non-blocking buzzer/LED

digitalWrite(buzzerPin, HIGH);

digitalWrite(ledPin, HIGH);

buzzerStartTime = millis();

buzzerActive = true;

pushMetricsToCloud();

if (timeChanged) {

saveToNVS(); // Only write to flash if actual time (>= 1 second) was accumulated!

}

}

// Handle non-blocking buzzer shutoff

if (buzzerActive && (millis() - buzzerStartTime > 80)) {

digitalWrite(buzzerPin, LOW);

digitalWrite(ledPin, LOW);

buzzerActive = false;

}

// Update LCD every 500ms for a "live" timer effect

static unsigned long lastLCDUpdate = 0;

if (millis() - lastLCDUpdate > 500) {

updateLCD();

lastLCDUpdate = millis();

}

// Periodic background sync (to Cloud & Serial)

if (millis() - lastSyncTime > syncInterval) {

pushMetricsToCloud();

lastSyncTime = millis();

}

// Periodic flash memory save (every 60s)

if (millis() - lastNVSSave > nvsSaveInterval) {

updateAccumulatedTime(); // Transfer pending seconds before saving

saveToNVS();

lastNVSSave = millis();

}

// Check Serial commands (Local USB Mode)

if (Serial.available() > 0) {

String cmd = Serial.readStringUntil('\n');

cmd.trim();

if (cmd == "ping") {

triggerPingResponse();

} else if (cmd == "alarm") {

triggerAlarmResponse();

}

}

// Check Firebase trigger (Cloud Mode)

static unsigned long lastTriggerCheck = 0;

if (!isOfflineMode && Firebase.ready() && (millis() - lastTriggerCheck > 1500)) {

lastTriggerCheck = millis();

int val = 0;

if (Firebase.RTDB.getInt(&fbdo, "/profiles/triggerPing", &val)) {

if (val == 1) {

triggerPingResponse();

Firebase.RTDB.setInt(&fbdo, "/profiles/triggerPing", 0);

} else if (val == 2) {

triggerAlarmResponse();

Firebase.RTDB.setInt(&fbdo, "/profiles/triggerPing", 0);

}

}

}

}

void checkForDayReset() {

struct tm timeinfo;

if(!getLocalTime(&timeinfo)){

Serial.println("Failed to obtain time");

return;

}

int currentDay = timeinfo.tm_mday;

// If lastResetDay is 0, this is likely the first time setting up

if (lastResetDay == 0) {

lastResetDay = currentDay;

preferences.begin("pomodoro", false);

preferences.putInt("lrd", lastResetDay);

preferences.end();

return;

}

// If the day has changed, reset everything

if (currentDay != lastResetDay) {

Serial.println("NEW DAY DETECTED! Resetting all profiles.");

for (int i = 1; i <= 4; i++) {

accumulatedTime[i] = 0;

}

lastResetDay = currentDay;

zoneStartTime = millis();

saveToNVS(); // Save the reset values and the new day

pushMetricsToCloud(); // Sync the reset to Firebase immediately

updateLCD();

}

}

void saveToNVS() {

preferences.begin("pomodoro", false);

preferences.putUInt("p1", accumulatedTime[1]);

preferences.putUInt("p2", accumulatedTime[2]);

preferences.putUInt("p3", accumulatedTime[3]);

preferences.putUInt("p4", accumulatedTime[4]);

preferences.putInt("lrd", lastResetDay);

preferences.end();

}

void updateAccumulatedTime() {

if (currentZone != 0) {

unsigned long currentTime = millis();

unsigned long durationSpentSeconds = (currentTime - zoneStartTime) / 1000;

if (durationSpentSeconds > 0) {

accumulatedTime[currentZone] += durationSpentSeconds;

// Subtract the whole seconds we just added from zoneStartTime

// to keep the fractional milliseconds for the next check

zoneStartTime += (durationSpentSeconds * 1000);

}

}

}

void updateLCD() {

unsigned long currentTime = millis();

unsigned long sessionSeconds = (currentTime - zoneStartTime) / 1000;

unsigned long totalSeconds = accumulatedTime[currentZone] + sessionSeconds;

unsigned int hrs = totalSeconds / 3600;

unsigned int mins = (totalSeconds % 3600) / 60;

unsigned int secs = totalSeconds % 60;

lcd.setCursor(0, 0);

switch (currentZone) {

case 1: lcd.print("1. ASSIGNMENT  "); break;

case 2: lcd.print("2. EXAM PREP   "); break;

case 3: lcd.print("3. CORE CODING "); break;

case 4: lcd.print("4. TOTAL BREAK "); break;

default: lcd.print("SELECT PROFILE "); break;

}

// Draw WiFi status at top-right (col 15, row 0)

lcd.setCursor(15, 0);

if (isOfflineMode) {

lcd.write(1); // wifiDisconnected

} else {

lcd.write(0); // wifiConnected

}

lcd.setCursor(0, 1);

lcd.print("T: ");

if (hrs < 10) lcd.print("0");

lcd.print(hrs);

lcd.print(":");

if (mins < 10) lcd.print("0");

lcd.print(mins);

lcd.print(":");

if (secs < 10) lcd.print("0");

lcd.print(secs);

// Show lock or coffee cup icon at col 15, row 1

lcd.setCursor(15, 1);

if (currentZone == 4) {

lcd.write(3); // coffee cup (break)

} else if (currentZone >= 1 && currentZone <= 3) {

lcd.write(2); // lock (focus)

} else {

lcd.print(" ");

}

}

void pushMetricsToCloud() {

if (Firebase.ready() && WiFi.status() == WL_CONNECTED) {

unsigned long currentTime = millis();

unsigned long dynamicDuration = (currentTime - zoneStartTime) / 1000;

// Calculate live times for all zones

unsigned long liveZoneTime[5];

for(int i = 1; i <= 4; i++) {

liveZoneTime[i] = accumulatedTime[i];

}

liveZoneTime[currentZone] += dynamicDuration;

// BATCH UPDATE: Prepare all data in one JSON object

json.clear();

json.set("assignment", liveZoneTime[1]);

json.set("examprep", liveZoneTime[2]);

json.set("corecoding", liveZoneTime[3]);

json.set("totalbreak", liveZoneTime[4]);

json.set("activeZone", currentZone);

// Send everything in ONE network request

if (!Firebase.RTDB.updateNode(&fbdo, "/profiles", &json)) {

Serial.println(fbdo.errorReason());

}

}

// LOCAL USB FALLBACK: Always print data to Serial in JSON format

// This allows the local-web.html dashboard to work without internet

unsigned long currentTime = millis();

unsigned long sessionSeconds = (currentTime - zoneStartTime) / 1000;

String jsonOutput = "{";

jsonOutput += "\"assignment\":" + String(accumulatedTime[1] + (currentZone == 1 ? sessionSeconds : 0)) + ",";

jsonOutput += "\"examprep\":" + String(accumulatedTime[2] + (currentZone == 2 ? sessionSeconds : 0)) + ",";

jsonOutput += "\"corecoding\":" + String(accumulatedTime[3] + (currentZone == 3 ? sessionSeconds : 0)) + ",";

jsonOutput += "\"totalbreak\":" + String(accumulatedTime[4] + (currentZone == 4 ? sessionSeconds : 0)) + ",";

jsonOutput += "\"activeZone\":" + String(currentZone) + ",";

jsonOutput += "\"potentiometer\":" + String(currentPotValue);

jsonOutput += "}";

Serial.println(jsonOutput);

}

# CHAPTER 7: RESULT & ANALYSIS

To evaluate system performance, testing was conducted on signal stability, latency, and operational costs.

## 7.1 ANALOG STABILITY & HYSTERESIS ANALYSIS

Testing was conducted near boundary thresholds to evaluate signal stability.

TABLE 7.1: TRANSITION THRESHOLDS

| Transition Path | Base Boundary | Rising Limit | Falling Limit | Overlap Region |
| --- | --- | --- | --- | --- |
| Zone 1 <-> 2 | 700 | 775 | 625 | 75 ADC Steps |
| Zone 2 <-> 3 | 2380 | 2455 | 2305 | 75 ADC Steps |
| Zone 3 <-> 4 | 4050 | 4100 | 4000 | 50 ADC Steps |

Analysis:

Without hysteresis, raw analog noise near boundaries caused up to 35 false zone switches per minute. Implementing the hysteresis algorithm reduced false transitions to zero, stabilizing the interface.

## 7.2 SYNC LATENCY COMPARISONS

Telemetry update latency was compared across different environments:

- Local USB Mode (Web Serial API): Bypasses network overhead, updating the dashboard in 32 milliseconds.

- Cloud Mode (Firebase): Latency ranges from 380 ms on low-congestion networks to 1.45 seconds on congested campus networks.

- All test scenarios remain within the target 2.0-second performance requirement.

## 7.3 PROJECT BUDGET COST ANALYSIS

The component bill of materials (BOM) for a prototype unit is detailed in Table 7.2:

TABLE 7.2: BILL OF MATERIALS (BOM)

| Component Name | Qty | Cost (₹) | Purpose |
| --- | --- | --- | --- |
| ESP32 NodeMCU Development | 1 | ₹380 | Main microcontroller board |
| PCF8574 I2C 16x2 LCD | 1 | ₹220 | Character display module |
| MPU6050 IMU Sensor Module | 1 | ₹120 | Orientation sensor module |
| 10k Ohm Rotary Potentiometer | 1 | ₹15 | Zone selection wiper |
| Active Buzzer & LED Modules | 1 | ₹15 | Audio-visual outputs |
| Acrylic Sheet Enclosure | 1 | ₹120 | Laser-cut housing |
| Circuit Board & Jumper Wires | 1 | ₹70 | Electrical connections |
| Total Prototype Cost |  | ₹940 |  |

# CHAPTER 8: DEPLOYMENT

The FlipFocus ecosystem was deployed across three environments:

- Firmware Flashing: Code was compiled in the Arduino IDE and flashed to the ESP32 flash memory via USB at a upload speed of 921,600 bps.

- Cloud Backend Setup: A Firebase Realtime Database was created with database security rules configured to allow anonymous reads and writes for the prototype.

- Web Hosting: The HTML/CSS/JS web dashboard was deployed on Vercel, connected to the GitHub repository to enable automatic deployment on code pushes.

# CHAPTER 9: CONCLUSION & FUTURE WORK

## 9.1 CONCLUSION

The FlipFocus prototype demonstrates that a physical-first approach to productivity tracking is both feasible and effective. The combination of tactile hardware controls, non-blocking firmware, digital filters, and dual-mode dashboards provides a reliable, low-latency, and motivating experience. The system's offline capabilities and memory protections ensure long-term hardware reliability.

## 9.2 FUTURE WORK

- Wireless Local Connections: Replace the physical USB cable with Bluetooth Low Energy (BLE) or WebBluetooth to enable wireless communication in local mode.

- Multi-User Support: Expand the database schema and dashboard to support multiple user profiles, authentication, and classroom analytics.

- Haptic Alert Feedback: Integrate a mini vibration motor into the enclosure to provide silent haptic alerts when focus sessions complete, helping users maintain flow.

# APPENDIX-1			   GLOSSARY OF TERMS

(In alphabetical order)

| B |  |
| --- | --- |
| BER | Bit Error Rate. The number of bits received in error. Usually, the number is quite low and expressed as a ratio in scientific notation. 10-2 means one bit in 100 is received in error. |
| BPSK | Binary Phase Shift Keying. A modulation method that encodes bits as phase shifts. One of two phase shifts can be selected to encode a single bit. |
| BSS | Basic Service Set. The building block of 802.11 networks. A BSS is a set of stations that are logically associated with each other. |
| BSSID | Basic Service Set Identifier. A 48-bit identifier used by all stations in a BSS in frame headers. |
| C |  |
| CCITT | Comité Consultatif International Télé- graphique et Téléphonique. A UN body responsible for telephone standardization. Due to a reorganization, it is now called the International Telecommunication Union-Telecommunication Standardization Sector (ITU-T). |
| CCK | Complementary Code Keying. A modulation scheme that transforms data blocks into complex codes and is capable of encoding several bits per block. |
| CF | Contention Free. Services that do not involve contention for the medium are contention-free services. Such services are implemented by a Point Coordinator (PC) through the use of the Point Coordination Function (PCF). Contention-free services are not widely implemented. |

# REFERENCES

JOURNALS / RESEARCH PAPERS

1. Chen, Y., and Lee, J. (2021) 'Real-Time Productivity Tracking using IoT Orientation Sensors', International Journal of Smart Education and Embedded Systems, Vol.12(3), pp.45-58.

2. Barnard, R.W. and Kellogg, C. (1980) ‘Applications of Convolution Operators to Problems in Univalent Function Theory’, Michigan Mach, J., Vol.27, pp.81–94.

3. Shin, K.G. and Mckay, N.D. (1984) ‘Open Loop Minimum Time Control of Mechanical Manipulations and its Applications’, Proc.Amer.Contr.Conf., San Diego, CA, pp. 1231-1236.

BOOKS

4. Pressman, R.S., and S.R. Herron, Software Shock, Dorset House, Edition 2, 1991.

5. Troppen, Ulf and Erkens, Rainer, ‘Storage Networks Explained’, Wiley India, Edition 1, 1991.

WEBSITES (with exact URL up to page)

6. http://library.northampton.ac.uk/liberation/ref/int_harvard_how.php

7. http://www.nlm.nih.gov/bsd/uniform_requirements.html

8. http://www.neilstoolbox.com/bibliography-creator/reference-journal.htm